Specification: software for STM32 Nucleo motor controller

- Introduction

A set of two BLDC motors must be controlled by a STM32 Nucleo-F103RB board. Required cycle see graph below:

- Both motor1 and motor 2 start at the same time and stop at the same time, both with the same given interval.

- Each motor must run at its own given constant speed

- A closed loop control (PID-control) is required to maintain the constant speed per motor at its target speed.

- BLDC motor specification

- Each BLCD motor has 4 wires

  - PWM signal with a frequency of 20kHz as an input

    - PWM control level “low”:		 ≤0.5V

    - PWM control level “high” 		2.7 … 5.5V

  - Tacho signal as an output

    - FG-output pulses: 6 pulses / revolution

    - FG-output pulse time “low”: 		50%

    - FG-output level “high” (no load): 	5 … 6V

    - FG-output level “low” (no load): 	<0.5V

  - + supply voltage (connected to external power supply)

  - – supply voltage (connected to external power supply)

- Software requirements for the STM32 Nucleo-F103RB board

- Programmed fixed in the C-program

  - One fixed PWM signal frequency setting of 20kHz for both motor 1 and motor 2

  - Target speed setting of 1900 rpm for motor 1

  - Target speed setting of 800 rpm for motor 2

  - PID parameter settings for motor 1

  - PID parameter settings for motor 2

  - One interval setting (time between start and stop) of 35 seconds for both motor 1 and motor 2

- Input 1: 	Start the interval with an input pulse HIGH (for a momentary push button)

- Input 2:	Tacho signal from motor 1

- Input 3:	Tacho signal from motor 2

- Output 1: 	PWM signal for motor 1

- Output 2: 	PWM signal for motor 2

- Plot speed of motor 1 and speed of motor 2 vs time elapsed using SWV data trace timeline graph in STM32CubeIDE

- Write cumulative rotations of motor 1 and motor 2 after completing a cycle in STM32CubeIDE

- Required scope of delivery

- Functional software for the STM32 Nucleo-F103RB board as described above

- Software suitable for uploading via STM32CubeIDE version 1.17.0 by us to our own STM32 Nucleo-F103RB board for testing the complete system with the motors and power supply.